WPT Deadband Database Support

# New Columns

- WPT_Nodes table:

  - IsDeadband (bit): 1 bit in the fwd packet

    - 0:  Conventional

    - 1:  Deadband

- WPT_FwdChannelData:

  - DeadbandSpan (tinyint): 0-30, 5 bits in the fwd packet

  - OccState (tinyint): 2 bits in the fwd packet

    - 0: No Support

    - 1: Occupied

    - 2: Unoccupied

    - 3: Ignore

- WPT_FCDHistory: Should contain these as it’s archive for FwdChannelData

  - DeadbandSpan (tinyint)

  - OccState (tinyint)

- WPT_BwdChannelData:

  - No change, but existing param2 and param3 are utilized for OccState & DBSpan / SPVlidFlag.

    - DeadbandSpan (tinyint): 5 bits in the bwd packet

    - OccState (tinyint): 2 bits in the bwd packet

    - SPValidFlag (bit): 1 bit in the bwd packet

      - 0: Valid

      - 1: Invalid

- WPT_NodeCurrentSetPoint: This should be reference data against the packet data

  - DeadbandSpan (tinyint)

  - OccState (tinyint)

- WPT_NodeMonitor: This data is a snapshot for convenience

  - DeadbandSpan (tinyint)

  - OccState (tinyint)

  - IsDeadband (bit)

- WPT_NodeGroupSetPoint

  - OccupiedDBSpan (tinyint)

  - UnoccupiedDBSpan (tinyint)

  - OccupiedDBTemp (smallint)

  - UnoccupiedDBTemp (smallint)

# Added Functions

- Udf_Split

  - Added to support RSSI min value searching

# Modified Stored Procedures

- Usp_InsertFwdChannelData:

  - Add DeadBandSpan and OccState value to FwdChannelData

  - Update IsDeadband value in Nodes based on DBSupport flag

- Usp_MoveTransDataToHistory

  - Add DeadBandSpan and OccState value to FCDHistory

  - Removed [WPT] prefix inside the SP

- Usp_GetGridViewData

  - Performance boost by only using temp table for data preparation (not using NodeMonitor)

  - Added WPT related data into the temp table

  - Added deadband conversion logic on getting the data

  - Support Traditional/WPT-DB/Mixed in one

- Usp_DailyPerformanceReport

  - Added deadband conversion logic on getting the data

  - Support Traditional/WPT-DB/Mixed in one

  - Update to return Routing path and RSSI information

- Usp_GetNodeAlarmDetails

  - Added deadband conversion logic on getting the data

  - Support Traditional/WPT-DB/Mixed in one

  - Update return Routing and RSSI information

- Usp_UpdateNodeGroupSetPoint

  - Added OccupiedDBSpan, UnoccupiedDBSpan, OccupiedDBTemp, UnoccupiedDBTemp parameters for new columns

- Usp_InsertBWDChannelData

  - From manual Setpoint change: Param1/Param2/Param3 for Setpoint/ OccState & DBSpan/VPValidFlag

  - Updated to use @Param2 value for OccState and DBSpan when updating WPT_NodeCurrentSetPoint before putting data into WPT_BwdChannelData

- Usp_FindAndResendBCD

  - Update to mark BCD to be resend for WPT-DB intelligent monitoring

  - Follow existing wait interval and setpoint tolerance rule (parameter)

- Usp_GetBCDDataForHub

  - This is for getting all the BWD channel data to send

  - Cleaned up

- Usp_GetProfileQueueData

  - Modified to return DBSpan related data

- Usp_SetProfileQueue

  - Removed ToDNodeVersion / ToDRepeaterVersion logic as it is not used but hindering scheduled set point logic

- Usp_InsertintoProfileQueue

  - Removed database name [WPT] binding in the query

- Usp_GetNodeGroupSetPoint

  - Updated to return DB related data (span and setpoint)

- Usp_GetNodeCommandDetails

  - Updated to return deadband flag

- Usp_GetNotificationInfo

  - Updated to get PhonePrefix data

- Usp_GetAlarmDataForNotofication

  - Update to get only zone temperature alarm

- Usp_UpdateDiagnosticData

  - Update to set alarm generation condition properly

- Usp_UpdateAcknowledgement

  - Update to reset alarm notification flag on ACKing

- usp_GetFwdChannelData

  - Update to get Deadband related data for ambient alarm checking

- Usp_GetGridViewNetworkData

  - Update to return RoutingInfo as expected format

- Usp_GetDataForServerUpload

  - Update to get the proper data format to upload to the Central Server

- Usp_RptGetAlarmDetails

  - Fixed returning ACK time as old date issue

- Usp_RptGetDataForGraph

  - Added to get graph data for passed node id(s)

- Usp_InsertSetPointForDRAP

  - Changed to support deadband setpoint for ADR

- Usp_UpdateFwdChannelDataStatus

  - Changed to support deadband BACnet setpoint object

  - Changed to support unoccupied BACnet flag object

- Usp_GetDiagnosticData

  - Changed to get more deadband related diagnostic data

  - Added Back Channel Data

- Usp_ToD

  - Changed to give precedence to scheduled setpoint change over DRAS event

- Usp_UpdateDefectiveNodeACK

  - Changed to support longer passed string values

- usp_RptGetOccAlarmDetails

  - Added more columns AlarmType, AlarmValue1. This sp is used by Zone Monitor->Report->After hour usage Report

# Added Stored Procedures

- Usp_GetNodeStatus

  - Added to support Advanced->Node Status View

- usp_RptGetNetworkDetails1

  - Added to support Setup->View Site Configuration

# Modified Views

- vwWPT_FwdChannelData

  - Add DeadBandSpan and OccState columns

# Modified Indexes

- WPT_FwdChannelData

  - _j_index_WPT_FwdChannelData_01

  - _j_index_WPT_FwdChannelData_02

  - _j_index_WPT_FwdChannelData_03

  - _j_index_WPT_FwdChannelData_04

  - _j_index_WPT_FwdChannelData_05

- WPT_FCDNetworkStatus

  - _j_index_WPT_FCDNetworkStatus_01

- WPT_FwdChannelRawData

  - _j_index_WPT_FwdChannelRawData_01

  - _j_index_WPT_FwdChannelRawData_02

- WPT_BwdChannelData

  - _j_index_WPT_BwdChannelData_01

  - _j_index_WPT_BwdChannelData_02

# Deleted Indexes

- idx_WPT_FwdChanneldata_OrgID

- idx_WPT_FwdChanneldata_StatusID

WPT Auto-Calibration Database Support

# New Table

- WPT_CalibrationStatus table:

  - ID  (int): index

  - NodeID (int): NodeID in decimal

  - Offset (decimal(4,2)): Calibration offset value

  - ControlPressure (decimal(4,2)): Reference pressure

  - CalibrationDate (datetime): command issued time

  - CalibrationStatusID (tinyint)

    - 0: SUCCESS

    - 1: PENDING

    - 2: FAILED_LIMIT_VIOLATION

    - 3: FAILED_INSTABILITY

    - 4: DB_NO_CALIBRATION

    - 5: PENDING_CLEAR

    - 6: CLEARED

    - 7: NO_NEED

    - 10: UNKNOWN

  - CurrentStatus

    - 0: new ongoing

    - 1: latest successful

    - 2: past

# New Columns

- WPT_FwdChannelData:

  - CalOffset (smalint): signed auto calibration offset value

# New Indexes

- _j_index_WPT_FwdChannelData_06

  - Added for BACnet operation efficiency

- _j_index_WPT_FwdChannelData_07

  - Added for BACnet operation efficiency

- _j_index_WPT_FCRDataHistory_01

  - Added for Diagnostic data export efficiency

- _j_index_WPT_FwdChannelData_08

  - Added for BACnet operation efficiency

- _j_index_WPT_NodeCurrentSetpoint_01

  - Added for BACnet operation efficiency

- _j_index_WPT_FCDHistory_02

  - Added for DailyPerformanceReport  efficiency

# Modified Stored Procedures

- Usp_DeleteNode:

- Usp_GetNodeStatus

- Usp_GetNodeAlarmDetails

- Usp_UpdateBCDCmdAckStatus

- Usp_MoveTransDataToHistory

  - Eventually reduce WPT_FwdChannelData table size moving old data to WPT_FCDHistory table

  - Moving count increased to 500 from 100 at a time

- Usp_InsertFwdChannelData

  - Added a check for existing RawDataID to prevent duplicate inserting

- Usp_InsertFCDRepeaterData

  - Added a check for existing RawDataID to prevent duplicate inserting

- Usp_GetFwdChannelData

  - Only returns active setpoints to process and mass updates instead of individual update

- Usp_UpdateFwdChannelDataStatus

  - Get rid of individual update causing deadlocks

- Usp_AddNodeGroup

  - Added to update default NodeGroup setpoint values

- Usp_GetProfileQueueData

  - Added to return default NodeGroup setpoint values

- Usp_FindAndUpdateDefectiveNode

  - Added BACnet update for defective node flag

# Added Stored Procedures

- Usp_GetCalibrationStatus:  Get the current data from WPT_CalibrationStatus table

- Usp_GetNodeCalibrationStatus: For a given node, get the necessary occupied temperature and pressure data with total period parameter

- Usp_InsertNodeCalibrationStatus:  Insert calculated result back to WPT_CalibrationStatus table

- Usp_Archive_WPT:  Move data from WPT database to WPT_Arch database

- Usp_AddNode: To support WPT type at the time of adding